IP Addressing & Subnetting Practice Worksheet

Computer Networks  |  Classful & Subnetted IP Address Problems (Class A, B & C)

Instructions: For each of the given IP address / prefix combinations below, determine the network address, subnet mask, number of host bits available, total number of addresses, number of usable host addresses, the first and last usable host IP addresses, and the broadcast address. Show your working.

| Question 1 |  | 192.168.1.0/24 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class C)

| Question 2 |  | 10.0.0.0/8 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class A)

| Question 3 |  | 172.16.0.0/16 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class B)

| Question 4 |  | 192.168.10.0/26 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class C)

| Question 5 |  | 10.10.0.0/16 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class A)

| Question 6 |  | 172.20.0.0/20 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class B)

| Question 7 |  | 200.100.50.0/28 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class C)

| Question 8 |  | 15.0.0.0/8 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class A)

| Question 9 |  | 150.50.0.0/16 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class B)

| Question 10 |  | 192.168.5.128/25 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class C)

| Question 11 |  | 172.31.0.0/12 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class B)

| Question 12 |  | 100.64.0.0/10 |
| --- | --- | --- |
| # | Find the following | Answer |
| 1 | Network Address |  |
| 2 | Subnet Mask |  |
| 3 | Host bits available |  |
| 4 | Total addresses |  |
| 5 | Usable hosts |  |
| 6 | First usable host IP |  |
| 7 | Last usable host IP |  |
| 8 | Broadcast address IP |  |

(Address belongs to Class A)

# Answer Key / Solutions

The table below provides complete solutions for all questions.

| Q | Given | Class | Network Addr. | Subnet Mask | Host Bits | Total Addresses | Usable Hosts | First Usable Host | Last Usable Host | Broadcast Addr. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 192.168.1.0/24 | C | 192.168.1.0 | 255.255.255.0 | 8 | 256 | 254 | 192.168.1.1 | 192.168.1.254 | 192.168.1.255 |
| 2 | 10.0.0.0/8 | A | 10.0.0.0 | 255.0.0.0 | 24 | 16,777,216 | 16,777,214 | 10.0.0.1 | 10.255.255.254 | 10.255.255.255 |
| 3 | 172.16.0.0/16 | B | 172.16.0.0 | 255.255.0.0 | 16 | 65,536 | 65,534 | 172.16.0.1 | 172.16.255.254 | 172.16.255.255 |
| 4 | 192.168.10.0/26 | C | 192.168.10.0 | 255.255.255.192 | 6 | 64 | 62 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 |
| 5 | 10.10.0.0/16 | A | 10.10.0.0 | 255.255.0.0 | 16 | 65,536 | 65,534 | 10.10.0.1 | 10.10.255.254 | 10.10.255.255 |
| 6 | 172.20.0.0/20 | B | 172.20.0.0 | 255.255.240.0 | 12 | 4,096 | 4,094 | 172.20.0.1 | 172.20.15.254 | 172.20.15.255 |
| 7 | 200.100.50.0/28 | C | 200.100.50.0 | 255.255.255.240 | 4 | 16 | 14 | 200.100.50.1 | 200.100.50.14 | 200.100.50.15 |
| 8 | 15.0.0.0/8 | A | 15.0.0.0 | 255.0.0.0 | 24 | 16,777,216 | 16,777,214 | 15.0.0.1 | 15.255.255.254 | 15.255.255.255 |
| 9 | 150.50.0.0/16 | B | 150.50.0.0 | 255.255.0.0 | 16 | 65,536 | 65,534 | 150.50.0.1 | 150.50.255.254 | 150.50.255.255 |
| 10 | 192.168.5.128/25 | C | 192.168.5.128 | 255.255.255.128 | 7 | 128 | 126 | 192.168.5.129 | 192.168.5.254 | 192.168.5.255 |
| 11 | 172.31.0.0/12 | B | 172.16.0.0 | 255.240.0.0 | 20 | 1,048,576 | 1,048,574 | 172.16.0.1 | 172.31.255.254 | 172.31.255.255 |
| 12 | 100.64.0.0/10 | A | 100.64.0.0 | 255.192.0.0 | 22 | 4,194,304 | 4,194,302 | 100.64.0.1 | 100.127.255.254 | 100.127.255.255 |

Quick reference — Default classful ranges: Class A: 1.0.0.0 – 126.255.255.255 (default mask /8, 255.0.0.0)  |  Class B: 128.0.0.0 – 191.255.255.255 (default mask /16, 255.255.0.0)  |  Class C: 192.0.0.0 – 223.255.255.255 (default mask /24, 255.255.255.0). Formulas used: Total addresses = 2^(host bits); Usable hosts = Total addresses − 2 (network & broadcast reserved, for prefixes /30 and shorter).